Source: https://en.wikipedia.org/wiki/MESI_protocol.

Cache coherency Protocols: These enforce the consistent view of shared memory blocks among all processor cores.

In multiprocessor systems, multiple processors may cache copies of the same memory block.

Why scenario arises?

- Updates by one processor must be seen by others. Solution: write update, write invalidate.

- When one process try to access old data(stale/expired) of other processors cache.

- In I/O operations(DMA), devices may update memory directly, bypassing caches.

To ensure correctness so, multiprocessor systems  hardware-based cache coherence protocols. These protocols coordinate multiple processors’ caches so that all copies of shared data remain consistent.

Broad classification:

Coherency protocols are classified into Snoopy based and Directory based based on how coherence information is tracked and enforced.

- Snoopy based coherence protocols: Here all caches monitor(or snoop) a shared communication medium (usually a bus) to observe the broadcasted memory transactions from other processors  to check if they have the copy of that block based on that it may update their copies or supply data directly if other core is requesting that block which present core have.

- Here coherence action action happens automatically and parallely.

- Examples: MSI, MESI, MOESI.

Pros: Simple, fast, low latency due to cache to cache communication.

Cons: Poor scalability(high power), More Bus utilization.

- Directory based Protocols: Here A centralized directory keeps info of which caches hold copies of each memory block, when the core sends memory access requests to directory, it coordinates the invalidations and updates only to relavent caches.

- Records in directory: which core has the cached copy, whether the block is shared or modified.

Pros:

No Broadcasting, scalable, reduces memory traffic.

Cons:

Higher latency due to directory lookups, additional storage overhead for directory entries.

MESI Protocol

It  is an invalidate-based protocol used for write-back caches(dirty bit set if data is modified than that of main memory). In the MESI protocol, if a cache miss happens then  cache-to-cache transfer possible if another cache holds the required block, reducing main memory Traffic.

Note: In MSI protocol, a cache miss would always go to main memory(bad approach).

States of Cache line: MESI protocol has 4-exclusive states(2-bits encoding), that a cache line can be marked with.

Modified(M-state):  Cache has the only valid copy; data is "dirty" (differs from main memory). This cache must write back on eviction,

If other processor/core issues a read request for the same block and goto S(shared) state.

- When the block is marked M(modified) or exclusive from shared, the copies of that block in other caches are marked as I(invalid).

Exclusive(E-state):

- The cache line is exclusive, Data is clean(same as MM), for read request from other cores, it changes to Shared state, for write request it changes to M- state.

Shared(S-state):

- Here, cache line is shared between multiple caches, it is read only state, data is consistent to the main memory, If any cache wants to write/update  must first invalidate other copies.

Invalied state(I-state):

- The cache line is not valid, any access result in cache miss.

MESI: Each cache line has MESI FSM changes its state b/w M-E-S-I depending on by requests from Processor/Shared Bus

- Processor Requests: These originate from the local processor accessing its own cache.

- PrRd: Processor requests to read a cache block.

- PrWr: Processor requests to write a cache block.

- Bus(snoop requests): These come from a shared bus issued by other processors monitored by snooping logic on the shared bus.

- BusRd(Bus Read): Another processor requests to read a cache block after it experiences cache miss.

  - The Response: Other caches check if they have the data. If a peer cache has it in the Modified state, it must provide the data. Otherwise, the data comes from main memory.

  - Result: The requesting cache moves to the Shared (if others have it) or Exclusive (if no one else has it) state.

- BusRdX(Bus read exclusive): After write miss, A processor requests to write to a cache block which another have.

- It broadcasts a BusRdX on the bus. Cache with Modified copy must supply data first to the requester and All other caches must invalidate their copies, Requesting cache gets exclusive ownership; all others transition to Invalid.

- BusUpgr(Bus upgrade): when a process updates a shared block in its cache it propagate that info to others for their invalidations.

- Flush(write back): A cache writes a modified block back to main memory.

- FlushOpt: A cache supplies a block directly to another cache (cache-to-cache transfer).

Note: Cache-to-cache transfers reduce read miss latency because fetching a block from another cache is usually faster than accessing main memory, especially in bus-based systems.

State Transition: Each cache block may be in any of the four states.

In a snooping system, all caches monitor every bus transaction continuousely via shared bus. Each cache tracks the state of the memory blocks it holds, and updates those states according to the coherence protocol (such as the MESI state diagram).

Snooping occurs at two points:

- Processor/cache side: Cache controllers snoop bus transactions to update cache line states.

- Memory side: The memory controller monitors transactions to maintain consistency with main memory.

Simplified Explanation of the MESI Example: Assume all processors access the same cache block, and all caches are initially empty. The request stream is:
R1, W1, R3, W3, R1, R3, R2
(where R = read, W = write, and the number is the processor ID). Initially assume all the caches are empty.

Key points:

- Snooping ensures all caches observe bus transactions and update states.

- Modified data is supplied by caches, not memory.

- BusUpgr avoids unnecessary data transfer by only invalidating copies.

- MESI efficiently maintains coherence while minimizing memory traffic.

Stpe by step behaviour:

- R1(Read by P1): P1 reads the block, causing a cache miss. The data is fetched from main memory and stored in P1’s cache in   			    the Exclusive (E) state.

- W1(write by P1): P1 writes to the block, which is a cache hit in the Exclusive state. The block is modified locally and moves to the Modified (M) state with no bus activity.

- R3: P3 reads the block and incurs a cache miss, issuing a BusRd. P1 snoops the request, flushes the modified data, and moves to Shared (S). P3 also receives the block in the Shared (S) state.

- W3: P3 writes to the block while in the Shared state. It issues a BusUpgr, causing P1 to invalidate its copy. P3 transitions to the Modified (M) state.

- R1: P1 reads again and misses, issuing a BusRd. P3 snoops the request, flushes the modified data, and both P1 and P3 move to the Shared (S) state. Main memory is updated.

- R3: P3 reads the block and hits in the Shared state, so no bus transaction occurs.

- R2: P2 reads the block and misses, issuing a BusRd. P1 and P3 snoop the request, one supplies the data, and all three caches end up in the Shared (S) state.

Read for ownership(RFO): A Read For Ownership is a cache coherence operation used when a processor wants to write to a cache line that is in the Shared (S) or Invalid (I) state. It combines a read with an invalidate broadcast, forcing all other caches to invalidate their copies of the line. The requesting processor gains exclusive access to the data and can safely modify it. In MESI, this operation is called BusRdX.

Performance issues in basic MESI:

A simple MESI implementation suffers from delays when.

- Writing to an invalid cache line(waiting for data to arrive).

- Processing invalidation messages from other processors.

To reduce these delays, processors use store buffers and invalidate queues.

Store buffers: When a processor writes to an invalid cache line, it issues an invalidate request and places the write into a store buffer instead of waiting for the cache line to arrive. The write is applied to the cache later. The processor can see its own pending writes in the store buffer, but other processors cannot until the data reaches the cache.

Invalidate Queue: Incoming invalidation requests are acknowledged immediately but processed later by placing them in an invalidate queue. Until processed, a cache line may still appear valid to the processor, even though it should be invalid.

Memory Barriers: Because store buffers and invalidate queues can delay visibility of writes.

- A store barrier forces pending writes in the store buffer to complete.

- A read barrier forces pending invalidations to be processed.

Memory barriers ensure correct ordering and visibility of memory operations across processors, and are necessary even in some single-threaded cases due to these buffering effects.

CODING

Tag Array: This module acts as the metadata storage (Tag Directory) for a cache. Its primary purpose is to keep track of which memory addresses are currently stored in the cache and what their coherency status is.

Role: In a typical operation, the cache controller compares tag_out with the incoming CPU address to determine if a Hit has 	occurred, while state_out tells the controller if it has permission to read or write that specific line.

Functions:

- Storage: Maintains 1024 entries, each containing a 20-bit Address Tag and a 2-bit MESI State (Modified, Exclusive, Shared, or Invalid).

- Indexing: Uses a 10-bit index to point to a specific cache line.

- Write logic: Synchronously updates the state and tag on the clock edge when write_en is high.

- Read logic: Provides an asynchronous (combinational) output of the stored metadata for immediate comparison by the cache controller.

module mesi_tag_array ( input clk, // Clock: Synchronizes metadata updates. input rst_n, // Reset: Active-low (used for logic initialization). // PORT INTERACTIONS: input [9:0] index, // From CPU/Controller: Selects the cache line to check/update // (derived from middle bits of the address). input [19:0] tag_in, // From CPU Address: The high-order address bits to be // stored as a "fingerprint" for future hit detection. input [1:0] state_in, // From MESI FSM: The new coherency state (M, E, S, I) // to be assigned to this cache line. input write_en, // From Cache Controller: High when a miss is resolved // or a state transition occurs (e.g., S -> M). output [19:0] tag_out, // To Comparator: The stored address bits used to // determine if a Hit has occurred. output [1:0] state_out // To FSM & Snooper: The current MESI status, used to // check permissions (e.g., "Is this line Invalid?"). ); // Metadata Storage: 1024 entries. Each entry = 2-bit State + 20-bit Tag. reg [21:0] storage [0:1023]; integer i; initial begin for (i = 0; i < 1024; i = i + 1) begin storage[i] = 22'b0; // Initialize all lines to 'Invalid' (2'b00) end end // Synchronous Write: Updates the tag and MESI state always @(posedge clk) begin if (write_en) storage[index] <= {state_in, tag_in}; end // Asynchronous Read: Provides state and tag for immediate hit/miss calculation assign {state_out, tag_out} = storage[index]; endmodule

Data Array: The Data Array is the "warehouse" of the cache. While the Tag Array stores the address "labels," this block stores the actual data values (payloads) retrieved from main memory, It interface primarily with the CPU, system bus, and the cache controller.

- index(from CPU/Controller): Decodes which of the 1024 cache lines is being accessed. It is usually derived from the middle bits of the CPU's memory address.

- wdata (Input Source):

  - From CPU: During a Write Hit, the processor updates the cached value.

  - From Bus/Memory: During a Cache Refill, data fetched from main memory is written into the array.

- rdata (Output Destination):

  - To CPU: On a Read Hit, the stored data is sent immediately to the processor.

  - To Bus: If a Modified (M) line is being evicted, this data is sent to the system bus for a Write-back to main memory.

- we (from Controller): The Controller pulls this high only when a valid write condition (Hit-Write or Refill-Update) is met.

module cache_data_array ( input clk, // Clock: Synchronizes writes to the data storage. // PORT INTERACTIONS: input [9:0] index, // From Cache Controller: Selects 1 of 1024 lines // (typically mapped from cpu_addr[11:2]). input [31:0] wdata, // From CPU (Write Hit) or System Bus (Memory Refill): // The 32-bit data to be stored in the cache. input we, // From FSM Controller: Logic high triggers a write // operation on the next clk edge. output [31:0] rdata // To CPU (Read Hit) or System Bus (Dirty Eviction/Write-back): // Provides combinational data output based on index. ); // 4KB Data Storage: 1024 entries of 32-bit words reg [31:0] data_ram [0:1023]; integer i; // Simulation-only initialization to zero initial begin for (i = 0; i < 1024; i = i + 1) begin data_ram[i] = 32'h0; end end // Synchronous Write: Updates cache content always @(posedge clk) begin if (we) data_ram[index] <= wdata; end // Asynchronous Read: Continuous output based on the current index assign rdata = data_ram[index]; endmodule

- MESI snooper: The MESI Snooper acts as a hardware-level monitor that maintains data consistency across multiple processors by enforcing the rules of the MESI coherency protocol. By continuously observing system bus transactions, it detects when a remote cache attempts to access a memory address currently held in the local cache, signaling a "snoop hit" if the tags match. Upon a hit, the snooper dictates necessary state transitions—such as downgrading a Modified or Exclusive line to Shared during a remote read, or to Invalid during a remote write—to ensure that no two caches hold conflicting versions of the same data. This mechanism is critical for preventing the CPU from processing "stale" data and for alerting the controller to perform a mandatory write-back if a remote processor requests data that is currently "dirty" in the local cache.

- module mesi_snooper ( - // PORT INTERACTIONS: - input [31:0] bus_addr, // From System Bus: The address being accessed - // by a remote processor. - input [1:0] bus_cmd, // From System Bus: Specifies if the remote access - // is a Read (2'b01) or a Write/Invalidate (2'b10). - input [19:0] local_tag, // From Tag Array: The address tag currently - // stored in this local cache line. - input [1:0] local_state, // From Tag Array: The current MESI state - // of this local cache line. - output reg [1:0] snoop_nxt_state, // To Tag Array/FSM: The required next state - // to maintain coherency after the snoop event. - output reg snoop_hit_o // To Cache Controller: High when this cache holds - // a valid copy of the address requested on the bus. - ); - // Conflict Detection: Match if bus address equals local tag and local line is not Invalid. - wire match = (bus_addr[31:12] == local_tag) && (local_state != 2'b00); - // Combinational Logic for State Transition - always @(*) begin - snoop_nxt_state = local_state; - snoop_hit_o = match; - if (match) begin - case (bus_cmd) - 2'b01: snoop_nxt_state = 2'b01; // Bus Read -> Transition to Shared (S) - 2'b10: snoop_nxt_state = 2'b00; // Bus Write -> Transition to Invalid (I) - default: snoop_nxt_state = local_state; - endcase - end - end - endmodule

- MESI FSM controller: The MESI FSM Controller serves as the central decision-making unit for cache coherency, translating local CPU memory requests into protocol-compliant state transitions. By evaluating the current_state of a cache line and the specific cpu_req, it determines the next_state (Modified, Exclusive, Shared, or Invalid) and generates the necessary bus_req_type to notify or invalidate other caches on the system bus. This combinational logic ensures that data remains consistent across a multi-core system, such as upgrading a Shared line to Modified during a write or choosing between Exclusive and Shared states based on the bus_shared_i signal during a read miss.

- module mesi_fsm_controller ( - // PORT INTERACTIONS: - input [1:0] cpu_req, // From CPU: The operation type (Read, Write, or No-op). - input [1:0] current_state, // From Tag Array: The existing MESI state of the addressed line. - input bus_shared_i, // From System Bus: Signal indicating if another cache - // also holds a copy of this data. - output reg [1:0] next_state, // To Tag Array: The updated MESI state after the CPU request. - output reg [1:0] bus_req_type // To Bus Controller: The command to broadcast to - // other caches (e.g., Read-Intent or Invalidate). - ); - // MESI State Encoding - localparam I = 2'b00, S = 2'b01, E = 2'b10, M = 2'b11; - // Request Type Encoding - localparam NOP = 2'b00, RD = 2'b01, WR = 2'b10; - // Combinational logic for protocol enforcement - always @(*) begin - // Default: Maintain current state and stay silent on the bus - next_state = current_state; - bus_req_type = NOP; - case (current_state) - // INVALID: Must fetch from memory/bus - I: if (cpu_req == RD) begin - // If others have it, go to Shared; otherwise, Exclusive - next_state = bus_shared_i ? S : E; - bus_req_type = RD; - end else if (cpu_req == WR) begin - // Write-through or Allocate: go straight to Modified - next_state = M; - bus_req_type = WR; - end - // SHARED: Line is clean but exists in other caches - S: if (cpu_req == WR) begin - // Must invalidate others before writing - next_state = M; - bus_req_type = WR; - end - // EXCLUSIVE: Line is clean and private - E: if (cpu_req == WR) begin - // Transition to Modified silently (no bus request needed) - next_state = M; - bus_req_type = NOP; - end - // MODIFIED: Line is dirty and private - M: begin - next_state = M; // Remain in Modified for any CPU hits - bus_req_type = NOP; - end - default: begin - next_state = I; - bus_req_type = NOP; - end - endcase - end - endmodule

- Cache Refill writeback: The Cache Refill/Write-Back Controller manages the critical handshake between the high-speed cache and the high-latency main memory or system bus. Its primary role is to bridge the timing gap by asserting a stable, registered mem_req signal whenever a cache miss is detected, maintaining this request until the memory subsystem signals completion via mem_ready. By using sequential logic, it ensures that memory requests are synchronized to the clock, providing the electrical stability necessary for signals traveling across physical bus lines or PCBs during data fetching or dirty line evictions.

- module cache_refill_write_back_ctrl ( - input clk, // Clock: Synchronizes the memory request signal. - input rst_n, // Reset: Active-low; clears outstanding memory requests. - // PORT INTERACTIONS: - input miss, // From Cache Controller: Asserted when a CPU request - // cannot be satisfied by the local cache. - input mem_ready, // From Main Memory/Bus: Signal indicating the external - // transaction (Read or Write) has successfully completed. - output reg mem_req // To Memory Controller/Bus: A registered request signal - // that initiates a bus cycle to fetch or store data. - ); - // Sequential logic to manage the memory handshake - always @(posedge clk or negedge rst_n) begin - if (!rst_n) - // Reset state: No pending requests - mem_req <= 1'b0; - else if (miss && !mem_ready) - // Request Phase: Keep mem_req high as long as a miss exists - // and memory hasn't responded. - mem_req <= 1'b1; - else - // Completion Phase: Drop mem_req once memory is ready or miss is cleared. - mem_req <= 1'b0; - end - endmodule

- Cache Core: The Cache Core serves as the top-level integration unit that orchestrates the interaction between the CPU, the local storage arrays, and the system bus. It binds the Tag and Data arrays with the MESI FSM, Snooper, and Refill Controller to form a complete coherent cache line. Its primary function is to prioritize and route requests: immediate servicing of CPU hits, initiating bus transactions for misses, and pre-empting local CPU access to handle high-priority snoop events from other processors to maintain system-wide data consistency.

- module cache_core_updated ( - input clk, // System Clock. - input rst_n, // Active-low Reset. - // CPU INTERFACE - input [31:0] cpu_addr, // Address requested by the CPU. - input [31:0] cpu_wdata, // Data to write (for Store operations). - input [1:0] cpu_op, // CPU Command: 00=NOP, 01=Read, 10=Write. - output [31:0] cpu_rdata, // Data returned to CPU (for Load operations). - output reg cpu_ready, // Handshake: High when operation is complete/data is valid. - // BUS INTERFACE - INPUT (SNOOPING) - input [31:0] bus_addr_in,// Address being accessed by another processor. - input [1:0] bus_cmd_in, // Command type from the bus (Read/Write). - input [31:0] bus_data_in,// Data arriving from memory (Refill). - // BUS INTERFACE - OUTPUT (DRIVING) - output reg [31:0] bus_addr_out, // Address we want to read/write on the bus. - output reg [1:0] bus_cmd_out, // Command we are sending to the bus. - output [31:0] bus_data_out, // Data we are flushing/writing back. - output bus_data_drive, // Control: Enable tri-state buffer to drive bus. - // CONTROL SIGNALS - input bus_grant, // Arbiter granted us bus access. - input mem_ready // Main memory finished the requested operation. - ); - // Internal wires for connecting sub-modules - reg [1:0] current_state, next_state; // State transitions for Tag Array - wire [19:0] tag_out; // Tag read from storage - wire [1:0] state_out; // MESI state read from storage - wire [31:0] data_out; // Data read from storage - // Control signals for internal Arrays - reg data_we; // Write Enable for Data Array - reg tag_we; // Write Enable for Tag Array - reg [31:0] data_wdata; // Data to be written to Data Array - // --- HIT/MISS LOGIC --- - // Hit = Tags match AND State is valid (not Invalid 00) - wire hit = (cpu_addr[31:12] == tag_out) && (state_out != 2'b00); - // Miss = CPU is asking for something (not NOP) AND it's not a Hit - wire miss = (cpu_op != 2'b00) && !hit; - // --- SUB-MODULE SIGNALS --- - wire mem_req_ctrl; // Request signal from Refill Controller - wire [1:0] fsm_next_state; // Next MESI state calculated by FSM - wire [1:0] fsm_bus_req; // Bus command required by FSM - wire bus_shared = 1'b0; // Hardcoded '0' for this simple example - wire [1:0] snoop_next_state; // Next MESI state calculated by Snooper - wire snoop_hit; // Flag: Another CPU is touching our data - // --- BUS DRIVING LOGIC --- - // Drive data bus ONLY if we have a Snoop Hit on a MODIFIED line (Flush dirty data) - assign bus_data_drive = snoop_hit && (state_out == 2'b11); - assign bus_data_out = data_out; - // --- INSTANTIATIONS --- - cache_refill_write_back_ctrl refill_ctrl ( - .clk(clk), .rst_n(rst_n), - .miss(miss && bus_grant), // Only start refill if we have the bus - .mem_ready(mem_ready), - .mem_req(mem_req_ctrl) - ); - mesi_fsm_controller fsm ( - .cpu_req(cpu_op), - .current_state(state_out), - .bus_shared_i(bus_shared), - .next_state(fsm_next_state), - .bus_req_type(fsm_bus_req) - ); - mesi_snooper snooper ( - .bus_addr(bus_addr_in), - .bus_cmd(bus_cmd_in), - .local_tag(tag_out), - .local_state(state_out), - .snoop_nxt_state(snoop_next_state), - .snoop_hit_o(snoop_hit) - ); - cache_data_array data_store ( - .clk(clk), - .index(cpu_addr[11:2]), // Use middle bits for Index - .wdata(data_wdata), - .we(data_we), - .rdata(data_out) - ); - mesi_tag_array tag_store ( - .clk(clk), .rst_n(rst_n), - .index(cpu_addr[11:2]), - .tag_in(cpu_addr[31:12]), // Use top bits for Tag - .state_in(next_state), - .write_en(tag_we), - .tag_out(tag_out), - .state_out(state_out) - ); - assign cpu_rdata = data_out; // Combinational read output to CPU - // --- MAIN CONTROLLER LOGIC --- - always @(*) begin - // Default values to prevent latch inference - next_state = state_out; - data_we = 1'b0; - tag_we = 1'b0; - data_wdata = 32'h0; - cpu_ready = 1'b0; - bus_addr_out = 32'h0; - bus_cmd_out = 2'b00; - // PRIORITY 1: SNOOPING (Maintain Coherency) - // If external bus activity matches our line, we must update state immediately. - if (snoop_hit && !bus_grant) begin - next_state = snoop_next_state; - tag_we = 1'b1; // Update Tag Array with new state (e.g., M -> I) - // Stall CPU if it tries to access while we are snooping - cpu_ready = (cpu_op == 2'b00); - end - // PRIORITY 2: CACHE HIT (Fast Path) - // Serve CPU immediately if data is present and valid. - else if (hit) begin - cpu_ready = 1'b1; - if (cpu_op == 2'b10) begin // CPU Write - data_we = 1'b1; // Write to Data Array - data_wdata = cpu_wdata; - next_state = fsm_next_state; // Update MESI state (e.g., S -> M) - tag_we = 1'b1; - // Inform bus if transitioning (e.g., Shared -> Modified requires broadcast) - bus_addr_out = cpu_addr; - bus_cmd_out = fsm_bus_req; - end - end - // PRIORITY 3: CACHE MISS (Slow Path) - // Fetch data from memory. - else if (miss && bus_grant) begin - bus_addr_out = cpu_addr; - bus_cmd_out = fsm_bus_req; // Send Read command to Bus - if (mem_ready) begin - // Memory has returned data; write it to cache - data_we = 1'b1; - data_wdata = bus_data_in; // Store fetched data - next_state = fsm_next_state; - tag_we = 1'b1; // Store new Tag and State (E or S) - cpu_ready = 1'b1; // Release CPU - end - end - // IDLE STATE - else if (cpu_op == 2'b00) begin - cpu_ready = 1'b1; - end - end - endmodule

- MESI Bus arbiter: The Bus Arbiter acts as the "traffic cop" of the multi-core system, preventing data collisions by ensuring that only one cache controller can drive the shared system bus at any given time. It receives request signals (req) from multiple cores and grants access (gnt) based on a specific scheduling policy—typically Round-Robin to ensure fairness and prevent starvation. By managing these permissions, the arbiter serializes memory requests, allowing the Snooper in each core to correctly observe and react to a single, stable transaction on the bus before the next one begins.

- module bus_arbiter ( - input clk, // System Clock. - input rst_n, // Active-low Reset. - // PORT INTERACTIONS: - input req_core0, // From Cache Core 0: High when Core 0 needs the bus - // (e.g., for a Refill or Write-back). - input req_core1, // From Cache Core 1: High when Core 1 needs the bus. - output reg gnt_core0, // To Cache Core 0: Grants permission to drive - // address/command signals onto the bus. - output reg gnt_core1 // To Cache Core 1: Grants permission to drive - // address/command signals onto the bus. - ); - // State Encoding for the Round-Robin FSM - localparam IDLE = 2'b00; - localparam GNT0 = 2'b01; // Core 0 has the bus - localparam GNT1 = 2'b10; // Core 1 has the bus - reg [1:0] state, next_state; - // Sequential Logic: State Update - always @(posedge clk or negedge rst_n) begin - if (!rst_n) state <= IDLE; - else state <= next_state; - end - // Combinational Logic: Next State & Output Generation - always @(*) begin - // Defaults - next_state = state; - gnt_core0 = 1'b0; - gnt_core1 = 1'b0; - case (state) - IDLE: begin - // If Core 0 requests, grant it (Priority to 0 in Idle, or random) - if (req_core0) next_state = GNT0; - else if (req_core1) next_state = GNT1; - end - GNT0: begin - gnt_core0 = 1'b1; // Assert Grant for Core 0 - // If Core 0 is done (req drops), check Core 1 (Fairness) - if (!req_core0) begin - if (req_core1) next_state = GNT1; - else next_state = IDLE; - end - end - GNT1: begin - gnt_core1 = 1'b1; // Assert Grant for Core 1 - // If Core 1 is done (req drops), check Core 0 (Fairness) - if (!req_core1) begin - if (req_core0) next_state = GNT0; - else next_state = IDLE; - end - end - default: next_state = IDLE; - endcase - end - endmodule

- MESI Top wrapper: The mesi_top module serves as the system-level integration layer for a dual-core coherent cache system. It orchestrates the communication between two independent cache cores, a shared system bus, and a main memory model using the MESI protocol. By incorporating a Bus Arbiter, the module serializes memory access requests, ensuring that only one core drives the bus at a time while the other core "snoops" the traffic to maintain data consistency. This top-level structure implements the complete infrastructure required for multi-core synchronization, including bus multiplexing for addresses and commands, as well as a shared data path where data can be supplied by either the main memory or a peer cache holding a "Modified" copy of the requested line.

- module mesi_top ( - input clk, - input rst_n, - // Core 0 CPU Interface: Connects to the primary processor 0 - input [31:0] cpu0_addr, - input [31:0] cpu0_wdata, - input [1:0] cpu0_op, // 00=NOP, 01=RD, 10=WR - output [31:0] cpu0_rdata, - output cpu0_ready, - // Core 1 CPU Interface: Connects to the primary processor 1 - input [31:0] cpu1_addr, - input [31:0] cpu1_wdata, - input [1:0] cpu1_op, - output [31:0] cpu1_rdata, - output cpu1_ready - ); - // Bus signals: The shared medium for all cache coherency traffic - wire [31:0] bus_addr; // Current active address on the bus - wire [1:0] bus_cmd; // Current active command (Read/Write) on the bus - wire [31:0] bus_data_wire; // Shared data line (Bidirectional logic) - wire bus_data_driven; // Status flag: Is anyone currently driving the data bus? - // Arbitration signals: Managing bus ownership - wire req0, req1; // Requests from Core 0 and Core 1 - wire grant0, grant1; // Permission granted by the Arbiter - // Memory interface: Connection to the slow main storage - wire mem_req; // Request to trigger a memory cycle - wire mem_ready; // Signal from memory that data/write is complete - wire [31:0] mem_data; // Data output from the memory model - // Core 0 outputs to bus: Internal wires for Core 0's bus contributions - wire [31:0] core0_bus_addr; - wire [1:0] core0_bus_cmd; - wire [31:0] core0_bus_data_out; - wire core0_drives_data; // High if Core 0 is performing a "Flush" (HitM) - // Core 1 outputs to bus: Internal wires for Core 1's bus contributions - wire [31:0] core1_bus_addr; - wire [1:0] core1_bus_cmd; - wire [31:0] core1_bus_data_out; - wire core1_drives_data; // High if Core 1 is performing a "Flush" (HitM) - // Bus multiplexing: Selects which core's address/command is broadcasted based on Arbiter grants - assign bus_addr = grant0 ? core0_bus_addr : (grant1 ? core1_bus_addr : 32'h0); - assign bus_cmd = grant0 ? core0_bus_cmd : (grant1 ? core1_bus_cmd : 2'b00); - // Bus data handling: Resolves who drives the data line (Memory vs. a Core with dirty data) - assign bus_data_driven = core0_drives_data ¦ core1_drives_data ¦ mem_ready; - assign bus_data_wire = core0_drives_data ? core0_bus_data_out : - (core1_drives_data ? core1_bus_data_out : - (mem_ready ? mem_data : 32'h0)); - // Request signals: A core requests the bus if it has a valid operation and is not yet "ready" (Cache Miss) - assign req0 = (cpu0_op != 2'b00) && !cpu0_ready; - assign req1 = (cpu1_op != 2'b00) && !cpu1_ready; - // Memory request: Triggered when the current bus master (grant holder) needs an external access - assign mem_req = (grant0 && (cpu0_op != 2'b00)) ¦¦ (grant1 && (cpu1_op != 2'b00)); - // Bus Arbiter: Serializes bus access between the two cores - bus_arbiter arbiter ( - .clk(clk), - .rst_n(rst_n), - .req0(req0), - .req1(req1), - .grant0(grant0), - .grant1(grant1) - ); - // Cache Core 0: Instance for the first processor - cache_core_updated core0 ( - .clk(clk), - .rst_n(rst_n), - .cpu_addr(cpu0_addr), - .cpu_wdata(cpu0_wdata), - .cpu_op(cpu0_op), - .cpu_rdata(cpu0_rdata), - .cpu_ready(cpu0_ready), - .bus_addr_in(bus_addr), // Listens (Snoops) to the shared bus - .bus_cmd_in(bus_cmd), - .bus_data_in(bus_data_wire), - .bus_addr_out(core0_bus_addr),// Proposes its address if it wins grant - .bus_cmd_out(core0_bus_cmd), - .bus_data_out(core0_bus_data_out), - .bus_data_drive(core0_drives_data), - .bus_grant(grant0), - .mem_ready(mem_ready) - ); - // Cache Core 1: Instance for the second processor - cache_core_updated core1 ( - .clk(clk), - .rst_n(rst_n), - .cpu_addr(cpu1_addr), - .cpu_wdata(cpu1_wdata), - .cpu_op(cpu1_op), - .cpu_rdata(cpu1_rdata), - .cpu_ready(cpu1_ready), - .bus_addr_in(bus_addr), // Listens (Snoops) to the shared bus - .bus_cmd_in(bus_cmd), - .bus_data_in(bus_data_wire), - .bus_addr_out(core1_bus_addr), - .bus_cmd_out(core1_bus_cmd), - .bus_data_out(core1_bus_data_out), - .bus_data_drive(core1_drives_data), - .bus_grant(grant1), - .mem_ready(mem_ready) - ); - // Simple Memory Model: Simulates the main system memory responding to bus requests - simple_memory memory ( - .clk(clk), - .rst_n(rst_n), - .mem_req(mem_req), - .mem_addr(bus_addr), - .mem_wdata(bus_data_wire), - .mem_we(bus_cmd == 2'b10), // Memory write if bus command is 'WR' - .mem_rdata(mem_data), - .mem_ready(mem_ready) - ); - endmodule.

- Testbench:

- `timescale 1ns/1ps - // minimum I should run this testbench for 5-6 us. - module mesi_tb; - // Clock and Reset - reg clk; - reg rst_n; - // Core 0 Interface - reg [31:0] cpu0_addr; - reg [31:0] cpu0_wdata; - reg [1:0] cpu0_op; - wire [31:0] cpu0_rdata; - wire cpu0_ready; - // Core 1 Interface - reg [31:0] cpu1_addr; - reg [31:0] cpu1_wdata; - reg [1:0] cpu1_op; - wire [31:0] cpu1_rdata; - wire cpu1_ready; - // Operation codes - localparam NOP = 2'b00; - localparam RD = 2'b01; - localparam WR = 2'b10; - // Test control - integer test_num; - integer errors; - // DUT instantiation - mesi_top dut ( - .clk(clk), - .rst_n(rst_n), - .cpu0_addr(cpu0_addr), - .cpu0_wdata(cpu0_wdata), - .cpu0_op(cpu0_op), - .cpu0_rdata(cpu0_rdata), - .cpu0_ready(cpu0_ready), - .cpu1_addr(cpu1_addr), - .cpu1_wdata(cpu1_wdata), - .cpu1_op(cpu1_op), - .cpu1_rdata(cpu1_rdata), - .cpu1_ready(cpu1_ready) - ); - // Clock generation - initial begin - clk = 0; - forever #5 clk = ~clk; - end - // Initialize signals - task initialize; - begin - cpu0_addr = 32'h0; - cpu0_wdata = 32'h0; - cpu0_op = NOP; - cpu1_addr = 32'h0; - cpu1_wdata = 32'h0; - cpu1_op = NOP; - test_num = 0; - errors = 0; - end - endtask - // Reset task - task reset_system; - begin - rst_n = 0; - initialize(); - repeat(5) @(posedge clk); - rst_n = 1; - repeat(2) @(posedge clk); - $display("=== System Reset Complete ==="); - end - endtask - // Wait for ready - task wait_ready_core0; - begin - while (!cpu0_ready) @(posedge clk); - @(posedge clk); - end - endtask - task wait_ready_core1; - begin - while (!cpu1_ready) @(posedge clk); - @(posedge clk); - end - endtask - // Core 0 Read - task core0_read(input [31:0] addr); - begin - cpu0_addr = addr; - cpu0_op = RD; - @(posedge clk); - wait_ready_core0(); - cpu0_op = NOP; - $display("[%0t] Core0 READ addr=0x%h data=0x%h", $time, addr, cpu0_rdata); - end - endtask - // Core 0 Write - task core0_write(input [31:0] addr, input [31:0] data); - begin - cpu0_addr = addr; - cpu0_wdata = data; - cpu0_op = WR; - @(posedge clk); - wait_ready_core0(); - cpu0_op = NOP; - $display("[%0t] Core0 WRITE addr=0x%h data=0x%h", $time, addr, data); - end - endtask - // Core 1 Read - task core1_read(input [31:0] addr); - begin - cpu1_addr = addr; - cpu1_op = RD; - @(posedge clk); - wait_ready_core1(); - cpu1_op = NOP; - $display("[%0t] Core1 READ addr=0x%h data=0x%h", $time, addr, cpu1_rdata); - end - endtask - // Core 1 Write - task core1_write(input [31:0] addr, input [31:0] data); - begin - cpu1_addr = addr; - cpu1_wdata = data; - cpu1_op = WR; - @(posedge clk); - wait_ready_core1(); - cpu1_op = NOP; - $display("[%0t] Core1 WRITE addr=0x%h data=0x%h", $time, addr, data); - end - endtask - // Check data - task check_data(input [31:0] expected, input [31:0] actual, input [7:0] core_id); - begin - if (expected !== actual) begin - $display("ERROR: Core%0d - Expected 0x%h, Got 0x%h", core_id, expected, actual); - errors = errors + 1; - end else begin - $display("PASS: Core%0d - Data matched: 0x%h", core_id, actual); - end - end - endtask - // Main test sequence - initial begin - $display("\n"); - $display("========================================"); - $display(" MESI Protocol Testbench Starting "); - $display("========================================\n"); - // Initialize - reset_system(); - // ==================================================================== - // TEST 1: Simple Read Miss - Core 0 - // ==================================================================== - test_num = 1; - $display("\n--- TEST %0d: Core0 Read Miss (I->E transition) ---", test_num); - core0_read(32'h0000_1000); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 2: Read Hit - Core 0 - // ==================================================================== - test_num = 2; - $display("\n--- TEST %0d: Core0 Read Hit ---", test_num); - core0_read(32'h0000_1000); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 3: Write Hit - Core 0 (E->M transition) - // ==================================================================== - test_num = 3; - $display("\n--- TEST %0d: Core0 Write Hit (E->M transition) ---", test_num); - core0_write(32'h0000_1000, 32'hDEADBEEF); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 4: Read after Write - Core 0 - // ==================================================================== - test_num = 4; - $display("\n--- TEST %0d: Core0 Read after Write ---", test_num); - core0_read(32'h0000_1000); - check_data(32'hDEADBEEF, cpu0_rdata, 0); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 5: Write Miss - Core 1 (I->M transition) - // ==================================================================== - test_num = 5; - $display("\n--- TEST %0d: Core1 Write Miss (I->M transition) ---", test_num); - core1_write(32'h0000_2000, 32'hCAFEBABE); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 6: Cache Coherence - Core1 reads what Core0 wrote - // ==================================================================== - test_num = 6; - $display("\n--- TEST %0d: Cache Coherence Test (Core1 reads Core0's data) ---", test_num); - core1_read(32'h0000_1000); - check_data(32'hDEADBEEF, cpu1_rdata, 1); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 7: Both cores read same location (Shared state) - // ==================================================================== - test_num = 7; - $display("\n--- TEST %0d: Both cores read same location ---", test_num); - core0_read(32'h0000_3000); - repeat(5) @(posedge clk); - core1_read(32'h0000_3000); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 8: Core0 writes to shared location (S->M, invalidate Core1) - // ==================================================================== - test_num = 8; - $display("\n--- TEST %0d: Core0 writes to shared location (S->M) ---", test_num); - core0_write(32'h0000_3000, 32'h12345678); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 9: Core1 reads updated value - // ==================================================================== - test_num = 9; - $display("\n--- TEST %0d: Core1 reads updated value ---", test_num); - core1_read(32'h0000_3000); - check_data(32'h12345678, cpu1_rdata, 1); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 10: Multiple writes to same location - // ==================================================================== - test_num = 10; - $display("\n--- TEST %0d: Multiple consecutive writes ---", test_num); - core0_write(32'h0000_4000, 32'hAAAAAAAA); - repeat(3) @(posedge clk); - core0_write(32'h0000_4000, 32'hBBBBBBBB); - repeat(3) @(posedge clk); - core0_read(32'h0000_4000); - check_data(32'hBBBBBBBB, cpu0_rdata, 0); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 11: Different cache lines - no interference - // ==================================================================== - test_num = 11; - $display("\n--- TEST %0d: Different cache lines ---", test_num); - core0_write(32'h0000_5000, 32'h11111111); - repeat(3) @(posedge clk); - core1_write(32'h0000_6000, 32'h22222222); - repeat(5) @(posedge clk); - core0_read(32'h0000_5000); - check_data(32'h11111111, cpu0_rdata, 0); - repeat(3) @(posedge clk); - core1_read(32'h0000_6000); - check_data(32'h22222222, cpu1_rdata, 1); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 12: Bus arbitration test (Core 0 priority) - // ==================================================================== - test_num = 12; - $display("\n--- TEST %0d: Bus arbitration (simultaneous requests) ---", test_num); - // Both cores request at same time - Core0 should get priority - fork - core0_read(32'h0000_7000); - core1_read(32'h0000_8000); - join - repeat(10) @(posedge clk); - // ==================================================================== - // TEST 13: Read-Modify-Write sequence - // ==================================================================== - test_num = 13; - $display("\n--- TEST %0d: Read-Modify-Write sequence ---", test_num); - core0_read(32'h0000_9000); - repeat(3) @(posedge clk); - core0_write(32'h0000_9000, cpu0_rdata + 32'h100); - repeat(3) @(posedge clk); - core0_read(32'h0000_9000); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 14: Cache line replacement scenario - // ==================================================================== - test_num = 14; - $display("\n--- TEST %0d: Same index different tag (conflict) ---", test_num); - core0_write(32'h0000_A000, 32'hAAAA0000); // Index 0x200 - repeat(5) @(posedge clk); - core0_write(32'h0010_A000, 32'hBBBB0000); // Same index, different tag - repeat(5) @(posedge clk); - core0_read(32'h0010_A000); - check_data(32'hBBBB0000, cpu0_rdata, 0); - repeat(5) @(posedge clk); - // ==================================================================== - // TEST 15: Stress test - alternating cores - // ==================================================================== - test_num = 15; - $display("\n--- TEST %0d: Alternating core accesses ---", test_num); - core0_write(32'h0000_B000, 32'h1111); - repeat(3) @(posedge clk); - core1_write(32'h0000_B000, 32'h2222); - repeat(3) @(posedge clk); - core0_write(32'h0000_B000, 32'h3333); - repeat(3) @(posedge clk); - core1_read(32'h0000_B000); - check_data(32'h3333, cpu1_rdata, 1); - repeat(5) @(posedge clk); - // Final summary - repeat(10) @(posedge clk); - $display("\n========================================"); - $display(" Test Summary "); - $display("========================================"); - $display("Total Tests: %0d", test_num); - $display("Total Errors: %0d", errors); - if (errors == 0) begin - $display("*** ALL TESTS PASSED ***"); - end else begin - $display("*** SOME TESTS FAILED ***"); - end - $display("========================================\n"); - $finish; - end - // Timeout watchdog - initial begin - #100000; - $display("\nERROR: Simulation timeout!"); - $finish; - end - // Waveform dump - initial begin - $dumpfile("mesi_tb.vcd"); - $dumpvars(0, mesi_tb); - end - endmodule

- Simulaion: